## LAB 10 – IMAGE COMPRESSION

### Objective

The objective of this laboratory experiment is to implement image compression techniques using Huffman Coding and JPEG-like Discrete Cosine Transform (DCT) compression. The performance of these techniques is evaluated using Compression Ratio (CR), Mean Square Error (MSE), Peak Signal-to-Noise Ratio (PSNR), and Rate Distortion (RD).

### Theory

Image compression is a technique used to reduce the amount of data required to represent an image without significantly affecting its visual quality. Compression techniques are broadly categorized into lossless and lossy compression.

Huffman Coding is a lossless compression technique based on symbol frequency. It assigns shorter binary codes to frequently occurring symbols and longer codes to less frequent ones, ensuring no loss of information.

JPEG-like compression is a lossy technique based on the Discrete Cosine Transform (DCT). The image is divided into 8×8 blocks, transformed into the frequency domain, quantized, and then reconstructed using inverse DCT. This method achieves higher compression by discarding less significant frequency components.

### Methodology

- A grayscale image is selected as the input image.

- Huffman coding is applied by calculating pixel frequencies and generating a Huffman tree.

- The image is encoded using the generated Huffman codes.

- For DCT compression, the image is divided into 8×8 blocks.

- DCT is applied to each block followed by quantization.

- Inverse DCT is applied to reconstruct the compressed image.

- Performance metrics are calculated for evaluation.

### Performance Metrics

- Compression Ratio (CR): Ratio of original image size to compressed image size.

- Mean Square Error (MSE): Average squared difference between original and reconstructed image.

- Peak Signal-to-Noise Ratio (PSNR): Measures reconstructed image quality.

- Rate Distortion (RD): Bits required per pixel after compression.

### Results and Discussion

Huffman coding achieved lossless compression with no degradation in image quality but provided limited compression ratio. JPEG-like DCT compression resulted in significantly higher compression ratios with a small loss in image quality. The trade-off between compression efficiency and image quality was clearly observed.

### Conclusion

Both lossless and lossy image compression techniques were successfully implemented. Huffman coding is suitable for applications requiring perfect reconstruction, while DCT compression is efficient for reducing storage and transmission costs.